Raed Hassan

102262488

Traffic Intersection Report

June 9, 2020

Project description

The goal of this project is to design a control circuit for a T_intersection by hardware description language (VHDL). The East-West road on the top of the T is a priority road and other road and pedestrian are forbidden until any buttons pushed.

Description of control circuit performance

The control circuit has three inputs, including two pedestrian buttons and a car detection sensor. The east-west road is always allowed to pass until one of the entrances turns on. When a circuit input is turned on, the lights turn on and off properly.

1-If a Car is detected, then the traffic lights will cycle to Green for the side road for an interval.

2-If a North-South Pedestrian is detected, then the lights will cycle so that the appropriate Walk light is displayed for a short interval.

3-If an East-West Pedestrian is detected, then the appropriate walk light will be displayed for a short interval. In each of the above cases, it is assumed that the east-west road is always green.

To implement this circuit, we used a top module that has button inputs and a car sensor and reset and clock. The output consist of two signals that sets lights. 
To prevent using latches we use 3 signal in a clock sensitive process structure then synthesis tool locate flip-flop instead of latches in the circuit. There are 2 main component for timer and traffic state machine. This components added to design as separately modules. Counter_top and Traffic_state_machine. Amber_time, Car_time and ped_time. We set this to 10 for simulation but in the real world they are longer and different. It has Mealy-style outputs and asynchrony reset. Another module Traffic_state_machine. This module has input and output below.Inputs includes reset, clock, car_time, ped_time, amber_time, CarNS, PedEW, and PedNS .Outputs includes LightsEW, LightsNS, and a led to debug. This module has a process called ped_button_memory to save buttons. And a process for debugging. It shows present state encoding in leds.

The main process called comb_traffic and contain moor state machine.  Sensitive list of process consist of state, CarNS, PedEW_temp, pedNS_temp, car_time, ped_time, amber_time signals.

Simulation output waveform

State Machine Diagram

Block diagram of top-level structure

VHDL sources : top module

library IEEE;

use IEEE.STD_LOGIC_1164.all;

use IEEE.STD_LOGIC_ARITH.all;

use IEEE.STD_LOGIC_UNSIGNED.all;

entity Traffic is

port

(

--asycnron rest

Reset           : in STD_LOGIC;

Clock           : in STD_LOGIC;

-- 2 signals for debuge

debugLED        : out std_logic_vector(3 downto 0);

LEDs            : out std_logic_vector(2 downto 0);

-- Car and pedestrian buttons

CarNS           : in STD_LOGIC; -- Car on NS road

PedEW       : in STD_LOGIC; -- Pedestrian moving EW (crossing NS road)

PedNS           : in STD_LOGIC; -- Pedestrian moving NS (crossing EW road)

-- Light control

LightsEW        : out STD_LOGIC_VECTOR (1 downto 0);   -- controls EW lights

LightsNS        : out STD_LOGIC_VECTOR (1 downto 0)    -- controls NS lights

);

end entity;

architecture Behavioral of Traffic is

signal clear          : std_logic;

-- signals that will be connected between state machine and counter modules

-- When the counter reaches the target value, it becomes signal 1.

signal car_time     : std_logic;

signal ped_time    : std_logic;

signal amber_time   : std_logic;

-- signals for inputs after the flip flop is used

signal CarNS_sync   : STD_LOGIC;

signal PedEW_sync   : STD_LOGIC;

signal PedNS_sync   : STD_LOGIC;

begin

--Observe the timer status

debugLED(3) <= amber_time;

debugLED(2) <= car_time;

debugLED(1) <= ped_time;

synchroniser:

process (reset, clock)

begin

--reset  everything to start at 0 initially

if (reset = '1') then

PedEW_sync <= '0';

PedNS_sync <= '0';

CarNS_sync <= '0';

elsif rising_edge(clock) then

-- each rising edge of the clock needs to make sure that all inputs are synchronised together so we are assigning whenever an input was held or pressed to a flip flop that saves it and updates it every rising edge of the clock

-- PedEW is the original input while PedEW_sync is the signal pedEW being assigned to a flip flop for sync

PedEW_sync <= PedEW;

PedNS_sync <= PedNS;

CarNS_sync <= CarNS;

end if;

end process synchroniser;

-- Show reset status on FPGA LED

debugLed(0) <= Reset;

--instance counter moudole

theCounter:

entity work.counter_top

port map(

reset => reset,

clock => clock,

clear => clear,

car_time => car_time,

ped_time => ped_time,

amber_time => amber_time);

State_machine:

entity work.Traffic_state_machine

port map(

Reset => Reset, -- assigning ports from state machine to its equivalent in the top_level module as these will be part of both the counter and state machine

Clock => Clock,

clear => clear,

car_time => car_time,

ped_time => ped_time,

amber_time => amber_time,

CarNS => CarNS_sync,

PedEW => PedEW_sync,

PedNS => PedNS_sync,

-- Light control

LightsEW => LightsEW,

LightsNS => LightsNS,

LEDs     => LEDs);

end architecture;

VHDL sources : counter

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

-- counter

entity counter_top is

port(

reset       : in std_logic;

clock       : in std_logic;

clear       : in std_logic;

car_time    : out std_logic;

ped_time    : out std_logic;

amber_time  : out std_logic);

end entity;

architecture beh of counter_top is

signal   count : natural range 0 to 100; --

constant car_time_const      : natural := 10;

constant ped_time_const     : natural := 10;

constant amber_time_const   : natural := 10;

begin

process(reset, clock)

begin

if (reset = '1') then

count <= 0;

elsif rising_edge(clock) then -- if clear is detected due to traffic light time finishing then we also clear the counter (reseting it)

if (clear = '1') then

count <= 0;

else

count <= count + 1; -- if the time limit is not reached we will count up untill we reach and clear the time

end if;

end if;

end process;

Car_time   <= '1' when (count = car_time_const)      else '0'; -- continue counting the counter up until counter = car_time_const

Ped_time   <= '1' when (count = ped_time_const)      else '0';  -- continue counting the counter up until counter = ped_time_const

Amber_time <= '1' when (count = amber_time_const)    else '0'; -- continue counting the counter up until counter = amber_time_const

end architecture;

VHDL sources : Traffic_state_machine

library IEEE;

use IEEE.STD_LOGIC_1164.all;

use IEEE.STD_LOGIC_ARITH.all;

use IEEE.STD_LOGIC_UNSIGNED.all;

entity Traffic_state_machine is

port

(

Reset : in STD_LOGIC;

Clock : in STD_LOGIC;

Clear : out STD_LOGIC;

car_time : in std_logic;

ped_time : in std_logic;

amber_time : in std_logic;

-- Car and pedestrian buttons

CarNS : in STD_LOGIC; -- Car on NS road

PedEW : in STD_LOGIC; -- Pedestrian moving EW (crossing NS road)

PedNS : in STD_LOGIC; -- Pedestrian moving NS (crossing EW road)

-- Light control

LightsEW : out STD_LOGIC_VECTOR (1 downto 0); -- controls EW lights

LightsNS : out STD_LOGIC_VECTOR (1 downto 0); -- controls NS lights

--leds for debuge

LEDs     : out  std_logic_vector(2 downto 0)

);

end Traffic_state_machine;

architecture Behavioral of Traffic_state_machine is

-- Encoding for lights

constant RED         : std_logic_vector(1 downto 0) := "00";

constant AMBER       : std_logic_vector(1 downto 0) := "01";

constant GREEN   : std_logic_vector(1 downto 0) := "10";

constant WALK        : std_logic_vector(1 downto 0) := "11";

type state_type is (EW_Green, EW_Amber, NS_Green, NS_Amber, NS_walk, EW_Walk);

signal state, next_state : state_type;

signal pedEW_temp       : STD_LOGIC; -- signals that will each connect a flip flop to the state machine, recording whether the ped utton is saved as pressed or not pressed

signal pedNS_temp       : STD_LOGIC;

signal Clear_ped_NS         : STD_LOGIC; -- 2 other signals directly going from the state machine to the flip flops to record whether we need to reset

signal Clear_ped_EW      : STD_LOGIC;

begin

--LEDs(2) <= pedEW_temp;

--LEDs(1) <= pedNS_temp;

--LEDs(0) <= CarNS;

sync :

process (Reset, clock)

begin

if (Reset = '1') then

state <= EW_Green;

elsif rising_edge(clock) then

state <= next_state;

end if;

end process;

ped_button_memory :

process (Reset, clock)

begin

if (Reset = '1') then --REMEMBER PEDNS AND PED EW ARE THE ORIGINAL INPUTS, THEY WILL NOT BE DIRECTLY WRITTEN IN THE STATE MACHINE

pedNS_temp <= '0';

pedEW_temp <= '0';

elsif rising_edge(clock) then

if (PedNS = '1') then -- when the ped button is pressed we save it as a '1' and assigning it to pedns_temp until the sstate is changed due to pedestrian light stopping its light

pedNS_temp <= '1';

elsif (Clear_ped_NS = '1') then -- when we need to clear the ped button press we will go back to the initial pedNS_temp not being pressed

pedNS_temp <= '0';

end if;

if (PedEW = '1') then

pedEW_temp <= '1';

elsif (Clear_ped_EW = '1') then

PedEW_temp <= '0';

end if;

end if;

end process;

--Monitor the status of states

--debug

process (next_state)

begin

case next_state is

when   EW_Green =>

leds <= "000";

when   EW_Walk =>

leds <= "001";

when   EW_Amber =>

leds <= "010";

when   NS_Green =>

leds <= "011";

when   NS_walk =>

leds <= "100";

when   NS_amber =>

leds <= "101";

when  others    =>

leds <= "111";

end case;

end process;

--main procees

comb_traffic :

process (state,  CarNS, PedEW_temp, pedNS_temp, car_time, ped_time, amber_time)

begin

lightsNS <= Red; -- default traffic light state

LightsEW <= Red;

-- default time counter start

clear <= '0';

Clear_ped_NS <= '0';

clear_ped_EW <= '0';

next_state <= state;

case state is

when EW_Green =>

LightsEW <= GREEN;

if (pedNS_temp = '1' or CarNS = '1') then

next_state <= EW_Amber;

elsif (PedEW_temp = '1') then

next_state <= EW_Walk;

end if;

when EW_Walk =>

lightsEW <= Walk;

clear_ped_EW <= '1'; -- clearing the pedestrian button press

if (ped_time = '1') then

clear <= '1'; ---------- clear counter then go to next state

next_state <= EW_Green;

end if;

when EW_Amber =>

LightsEW <= Amber;

if (amber_time = '1') then --- amber time reached so reset counter and move to next state depending on whether the ped has pressed the button or not

clear <= '1';

if (pedNS_temp = '1') then

next_state <= NS_Walk;

else

next_state <= NS_Green;

end if;

end if;

when NS_Green =>

LightsNS <= Green;

if (car_time = '1') then

clear <= '1';-- reset counter

next_state <= NS_Amber;

end if;

when NS_walk =>

LightsNS <= Walk;

Clear_ped_NS <= '1'; -- clear

if (ped_time = '1') then

clear <= '1';-- reset counter

next_state <= EW_Green;

end if;

when NS_Amber =>

LightsNS <= Amber;

if (amber_time = '1') then

clear <= '1';

next_state <= EW_Green;

end if;

end case;

end process;

end architecture;

Synthesis report

Here are the important parts of the synthesis report and some explanations.

FMS optimizing with one-hot encoding. All signals saved in flip-flop and no latch insert in design. Maximum clock frequency: 194MHz and no gated clock exists is design. The important part is the color red.

=========================================================================
*                       Advanced HDL Synthesis                          *
=========================================================================

Analyzing FSM for best    encoding.
Optimizing FSM on signal with one-hot encoding.
----------------------
 State    | Encoding
----------------------
 ew_green | 000001
 ew_amber | 000010
 ns_green | 010000
 ns_amber | 100000
 ns_walk  | 001000
 ew_walk  | 000100
----------------------

=========================================================================
Advanced HDL Synthesis Report

Macro Statistics
# FSMs                                                 : 1
# Counters                                             : 1
 7-bit up counter                                      : 1
# Registers                                            : 5
 Flip-Flops                                            : 5

=========================================================================

=========================================================================
*                         Low Level Synthesis                           *
=========================================================================

Optimizing unit  ...

Optimizing unit  ...

Mapping all equations...
Building and optimizing final netlist ...
Found area constraint ratio of 100 (+ 5) on block Traffic, actual ratio is 2.

Final Macro Processing ...

=========================================================================
Final Register Report

Macro Statistics
# Registers                                            : 18
 Flip-Flops                                            : 18

=========================================================================

=========================================================================
*                           Partition Report                            *
=========================================================================

Partition Implementation Status
-------------------------------

  No Partitions were found in this design.

-------------------------------

=========================================================================
*                            Final Report                               *
=========================================================================
Final Results
RTL Top Level Output File Name     : Traffic.ngr
Top Level Output File Name         : Traffic
Output Format                      : NGC
Optimization Goal                  : Speed
Keep Hierarchy                     : No

Design Statistics
# IOs                              : 16

Cell Usage :
# BELS                             : 48
#      LUT2                        : 10
#      LUT2_L                      : 1
#      LUT3                        : 4
#      LUT3_D                      : 2
#      LUT3_L                      : 2
#      LUT4                        : 18
#      LUT4_D                      : 1
#      LUT4_L                      : 3
#      MUXF5                       : 6
#      VCC                         : 1
# FlipFlops/Latches                : 18
#      FDC                         : 12
#      FDCE                        : 5
#      FDP                         : 1
# Clock Buffers                    : 1
#      BUFGP                       : 1
# IO Buffers                       : 15
#      IBUF                        : 4
#      OBUF                        : 11
=========================================================================

Device utilization summary:
---------------------------

Selected Device : 3s50pq208-5 

 Number of Slices:                       22  out of    768     2%  
 Number of Slice Flip Flops:             18  out of   1536     1%  
 Number of 4 input LUTs:                 41  out of   1536     2%  
 Number of IOs:                          16
 Number of bonded IOBs:                  16  out of    124    12%  
 Number of GCLKs:                         1  out of      8    12%  

---------------------------
Partition Resource Summary:
---------------------------

  No Partitions were found in this design.

---------------------------


=========================================================================
TIMING REPORT

NOTE: THESE TIMING NUMBERS ARE ONLY A SYNTHESIS ESTIMATE.
      FOR ACCURATE TIMING INFORMATION PLEASE REFER TO THE TRACE REPORT
      GENERATED AFTER PLACE-and-ROUTE.

Clock Information:
------------------
-----------------------------------+------------------------+-------+
Clock Signal                       | Clock buffer(FF name)  | Load  |
-----------------------------------+------------------------+-------+
Clock                              | BUFGP                  | 18    |
-----------------------------------+------------------------+-------+

Asynchronous Control Signals Information:
----------------------------------------
-----------------------------------+------------------------+-------+
Control Signal                     | Buffer(FF name)        | Load  |
-----------------------------------+------------------------+-------+
Reset                              | IBUF                   | 18    |
-----------------------------------+------------------------+-------+

Timing Summary:
---------------
Speed Grade: -5

   Minimum period: 5.141ns (Maximum Frequency: 194.532MHz)
   Minimum input arrival time before clock: 1.572ns
   Maximum output required time after clock: 11.979ns
   Maximum combinational path delay: 6.856ns

Timing Detail:
--------------
All values displayed in nanoseconds (ns)

=========================================================================
Timing constraint: Default period analysis for Clock 'Clock'
  Clock period: 5.141ns (frequency: 194.532MHz)
  Total number of paths / destination ports: 202 / 20
-------------------------------------------------------------------------
Delay:               5.141ns (Levels of Logic = 4)
  Source:            theCount/count_2 (FF)
  Destination:       theCount/count_6 (FF)
  Source Clock:      Clock rising
  Destination Clock: Clock rising

  Data Path: theCount/count_2 to theCount/count_6
                                Gate     Net
    Cell:in->out      fanout   Delay   Delay  Logical Name (Net Name)
    ----------------------------------------  ------------
     FDC:C->Q              5   0.626   1.078  theCount/count_2 (theCount/count_2)
     LUT3_D:I0->O          5   0.479   0.806  theCount/Mcount_count_eqn_321 (N01)
     LUT4:I3->O            1   0.479   0.704  State_machine/Clear_SW10_F (N46)
     LUT4:I3->O            1   0.479   0.000  theCount/Mcount_count_eqn_6_G (N55)
     MUXF5:I1->O           1   0.314   0.000  theCount/Mcount_count_eqn_6 (theCount/Mcount_count_eqn_6)
     FDC:D                     0.176          theCount/count_6
    ----------------------------------------
    Total                      5.141ns (2.553ns logic, 2.588ns route)
                                       (49.7% logic, 50.3% route)

=========================================================================
Timing constraint: Default OFFSET IN BEFORE for Clock 'Clock'
  Total number of paths / destination ports: 3 / 3
-------------------------------------------------------------------------
Offset:              1.572ns (Levels of Logic = 1)
  Source:            PedEW (PAD)
  Destination:       PedEW_sync (FF)
  Destination Clock: Clock rising

  Data Path: PedEW to PedEW_sync
                                Gate     Net
    Cell:in->out      fanout   Delay   Delay  Logical Name (Net Name)
    ----------------------------------------  ------------
     IBUF:I->O             1   0.715   0.681  PedEW_IBUF (PedEW_IBUF)
     FDC:D                     0.176          PedEW_sync
    ----------------------------------------
    Total                      1.572ns (0.891ns logic, 0.681ns route)
                                       (56.7% logic, 43.3% route)


=========================================================================
Timing constraint: Default path analysis
  Total number of paths / destination ports: 1 / 1
-------------------------------------------------------------------------
Delay:               6.856ns (Levels of Logic = 2)
  Source:            Reset (PAD)
  Destination:       debugLED<0> (PAD)

  Data Path: Reset to debugLED<0>
                                Gate     Net
    Cell:in->out      fanout   Delay   Delay  Logical Name (Net Name)
    ----------------------------------------  ------------
     IBUF:I->O            19   0.715   1.233  Reset_IBUF (debugLED_0_OBUF)
     OBUF:I->O                 4.909          debugLED_0_OBUF (debugLED<0>)
    ----------------------------------------
    Total                      6.856ns (5.624ns logic, 1.233ns route)
                                       (82.0% logic, 18.0% route)

=========================================================================


Total REAL time to Xst completion: 6.00 secs
Total CPU  time to Xst completion: 5.67 secs
 
--> 

Total memory usage is 256944 kilobytes

Number of errors   :    0 (   0 filtered)
Number of warnings :    0 (   0 filtered)
Number of infos    :    0 (   0 filtered)